# Guidelines for sustainability in capital works Updates summary

# Summary of guideline changes

The following table provides a summary of the updated changes from the guidelines released in May 2021.

| Section | Change |
| --- | --- |
| Transitioning to all-electric facilities | No change to policy on all-electric buildings. Links transition to all-electric buildings with 100% renewable electricity commitment by 2025. Acknowledgement that it may not always be viable to deliver all-electric buildings on sites that already have gas. |
| Design targets: buildings eligible for NABERS ratings | No change to overall design target. Replacement of carbon and water design targets per OBD with a requirement to use the NABERS reverse calculator. |
| Design targets: buildings ineligible for NABERS ratings | No change to overall design target. Greater clarity on measuring building performance through percentage improvement of Section J though deemed to satisfy. No set percentage improvement requirements. |
| Waste management | No change to space allocations for waste infrastructure. Addition of requirement for sustainability consultant to work with health services to map waste flows, using a standard template, to assist with identifying necessary space allocation for waste. |
| Delivering climate resilient facilities | Addition of a requirement to prepare a draft Climate Adaptation Plan in line with GreenStar for sites that have a significant climate risk. |
| Section J compliance | Clarifies that deemed to satisfy can be used for section J compliance other than building sealing, which must use JV4 method (air tightness testing). At least 10% of the building is to be tested and standards not met, a second test area is required. Clarifies who is responsible for each component of section J and sustainability consultant must review documentation prior to submission. |

| Section | Change |
| --- | --- |
| Application of the guidelines on existing projects | Provided clarity that projects that were in SD or DD prior to 1 July 2020 should endeavour to meet the guidelines where the budget and design allows. |
| Quality assurance | Addition of sustainability control points in VHBA’s Project Management Framework. |